11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Date Prepared: 3/14/2011

Material Safety Data Sheet

Section 1: Product and Company Identification

Product Name:          Polyaspartic Clear Coat-Part A

Sold By:                    Slide-Lok

11803 N. Saguaro Blvd Ste. 11

Fountain Hills, AZ 85268

1(480) 222-9720

FOR CHEMICAL      Spill, Leak, Fire, Exposure,

EMERGENCY          or Accident, Call 24 hours

1-(480) 222-9720

Fax Phone:               1(480) 222-9711

Section 2: Composition/Information on Ingredients

Hazardous Components                                     CAS#           ACGIH TLV          OSHA PEL

1,6 Hexamethylene Diiscyanate                           822-06-0        N/A                      N/A

Benzene, 1-chloro-4-(Triflouromethyl)                  98-56-6          N/A                      N/A

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 3: Hazards Identification

Emergency Overview:

Routes of Entry:                    Route                 Entry Risk

Inhalation           Possible

Ingestion            Possible

Skin Contact       Possible

Eye Contact        Possible

Potential Health Effects: Inhalation          May cause nausea and respiratory tract irritation. May

cause sensitization in susceptible individuals.

Ingestion          No significant signs or symptoms of any adverse health                                       					 hazards.

Skin Contact     Product is a slight skin irritant. May cause swelling and redness.

Eye Contact      Mild eye irritant. Vapors slightly uncomfortable. Splashes              					 irritating and painful.

Acute Health Hazards:   No Information is available on the acute health hazards of this product. Based

upon data from testing of similar products, no significant effects are expected.

Chronic Health Hazards:  If misted or at high concentrations, may cause pallor, nausea, anesthetic or

narcotic effects.

Medical conditions           Skin and eye sensitization. Depending on individual skin sensitivity, chronic or Generally Aggravated	  prolonged exposure may result in irritation, blistering, burning and peeling of                   by Exposure:                    skin layers.

Carcinogenicity:               OSHO: No data    ACGIH: No data   NTP: No data   IARC: No data

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 4: First Aid Measures

Eye Contact:                    Immediately flush with plenty of water for at least 15 minutes. If redness, itching,              			  or a burning sensation develops, seek medical attention.

Skin Contact:                   Remove from skin immediately. Rinse with clean water for 20-30 minutes. Use                			  soapy water if needed. If redness, itching or a burning sensation develops, seek  			  medical attention.

Ingestion:                         Do NOT induce vomiting! Dilute with water and seek medical attention 					  immediately.

Inhalation:                       Move victim to fresh air immediately. Give oxygen and seek medical attention.

Section 5: Fire-Fighting Measures

Flammable Limits:         Not determined

Flash Point, (Method Used):    Approximately 42.8 C (109 F) Tag CC

Autoignition Temperature:      Approximately > 500 C

NFPA Hazard Rating:            Health: 2        Hazard scale     Protective Equipment

Flammability: 2        0=LEAST                 A=SAFETY GLASSES

Reactivity: 1        1=SLIGHT                B=SAFETY GLASSES, GLOVES

Other:           2=MODERATE        C=SAFETY GLASSES, GLOVES AND 								               APRON

HMIS Hazard Rating:         Health: 2           3=HIGH

Flammability: 2            4=EXTREME

Reactivity: 1

Protection: B

Extinguishing Media:         Foam/Carbon Dioxide/Dry Chemical/Water Fog

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Special Fire Fighting Procedures:       Remove all ignition sources. Wear self-contained breathing 						        apparatus and complete personal protective equipment when 					        entering confined areas where potential for exposure to vapors or 				                     products exist.

Unusual Fire and Explosion Hazards:     Closed containers may rupture due to build up of pressure when 					             exposed to extreme heat.

Section 6: Accidental Release Measures

Accidental Release Measures:   Avoid contact with material. Persons not wearing appropriate protective equipment should be excluded until spill is cleaned up. Stop spill at source, pump liquid to salvage container. Remaining liquid may be taken up on claw, diatomaceous earth, or other absorbent. Treat with Decontamination Solutions: Nonionic surfactant Union Carbide's Tergitol TMN-10(20%) and water(80%); concentrated ammonia(3-8%), detergent(2%) and water (90-95%).

Section 7: Handling and Storage

Handling & Storage Precautions:  Prevent all skin and eye contact. Avoid breathing vapors. Re-seal partially used containers. Wash with soap and water before eating or drinking. Protect from moisture contamination. Exothermic generation of carbon dioxide may cause dangerous pressure. Keep away from all ignitable sources as well as extreme heat. Do not expose to excessive moisture.

Section 8: Exposure Control/Personal Protection

Ventilation:  Adequate ventilation required. Local exhaust may be required in some areas. Special exhausting generally not required. Mechanical exhaust usually adequate.

Respiratory Protection:  Respiratory masks should be worn at all times when adequate ventilation does not exist. A NIOSH/MSHA respirator is acceptable.

Eye Protection:  Chemical tight goggles; full face shield if splashing is possible.

Skin Protection:  Avoid all skin contact.

Other Protective Clothing or equipment:  Use impervious gloves, neoprene or rubber.

Work/Hygienic Practices:  Good air flow in working area. Eyewash station and safety shower should be available. Gloves and respiratory equipment should be worn at all times.

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 9: Physical and Chemical Properties

Appearance:			Clear/Colorless

Odor:				Negligible

Physical State:			Liquid

ph as Supplied:		Not applicable

Boiling Point:			Begins at > 139 C (282 F)

Melting Point:			-36 C (-33 F)

Pour Point:			Approximately -51C (59.8F)

Vapor Pressure (mmHg):	= 7.5-5 mmHG @ 77 F for MDI

Specific Gravity (Water=1):	1.199 @ 20C (68 F)

Volatile Organic Compounds (VOC):	With water: None     Without Water: None

Solubility in Water:		Not Soluble (reacts to liberate Co2 gas)

Percent Volatile:		By weight: Negligible

Bulk Density:			9.98 Lb./Gal.

Molecular Weight:		500 Approximate Value, for the polyisocyanate

Section 10: Stability and Reactivity

Stability: Stable

Conditions to Avoid (Stability): Avoid excessive heat, open flame, sparks and strong oxidizing agents. Protect from atmospheric moisture. Replace outage with inert dry nitrogen.

Incompatibility (Materials to Avoid): Avoid water, acid, base (alkalis, ammonia), alcohols, metal compounds.

Hazardous Decomposition or Byproducts: Isocyanate vapors or mist, carbon dioxide, carbon monoxide, nitrogen oxides.

Hazardous Polymerization: May occur

Conditions to Avoid (Polymerization): Avoid incompatible reactants, especially strong bases, water or temperatures over 160 degrees centigrade.

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

Section 11: Toxicological Information

Toxicological Information (Acute Toxicity): ORAL LD50: Estimated to be greater than 10000 MG/KG (rats). DERMAL LD50: Estimated to be greater than 5000 MG/KG (rabbits). INHALATION LC50: Lower Respiratory (pulmonary) Irritant. LC 50 Value Range from 137-1150 MG/M3 were obtained in rats exposed to aerosols. (4H EXP.) Severe irritant capable of inducing corneal injury (rabbit). Maximum primary eye irritant score: 54.6/110 for 24 hour skin effects: Moderate and Dermal. Primary dermal irritation score: 3.4/8.0 (rabbit). Pulmonary and dermal sensitizer in animals and humans.

Section 12: Ecological Information

Ecological Information:  No Data

Section 13: Disposal Considerations

Waste Disposal Method: Dispose of according to current local, state, and federal regulations.

RCRA Hazard Class: Non-Regulated

Section 14: Transport Information

U.S. Department of Transportation

Proper Shipping Name: Polyisocyanate

Hazard Class: Non-Regulated in 55 gallon drums, NOI

ID Number: NA 3082

Packing Group: III

Section 15: Regulatory Information

U.S. Federal Regulations

TSCA Toxic Substance Control Act: Listed on the TSCA Inventory

CERC:A (Comprehensive Response Compensation and Liability Act: (40 CFR302): None

SARA Title III (Superfund Amendments and Reauthorization Act:

311/312 Hazard Categories: Immediate health hazard; Delayed health hazard; Reactive; Fire hazard.

313 Reportable Ingredients: None

11803 N Saguaro Blvd Ste 11    |    Fountain Hills, AZ 85268

RCRA Status: When discarded in its purchased form, this product meets the criteria of ignitability, and should be managed as a hazardous waste. (EPA Hazardous Waste Number D001) (40 CFR 261.20-24)

State Regulations: Homopolymer of HDI 28182-81-2, 83%, PA, NJ. Aromatic (Solvent NAPHTHA)(AR100) 6472-95-6, 4.6%, PA, NJ. N-Butyl Acetate (BA) 123-86-4, 4.6%, PA, NJ, MA. Acetate Ester 90438-79-2, 7.8%, PA, NJ. California Proposition 65 To the best of our knowledge, this product contains no levels of listed substances which the state of California has found to cause cancer, birth defects or other reproductive effects.

Section 16: Other Information

Disclaimer: This information is formulated for use by elements of the Department of Defense. The United States of America in no manner whatsoever expressly or implied warrants, states, or intends said information to have any application, use or viability  by or to any person or persons outside the Department of Defense nor any person or persons contracting with any instrumentality of the United States of America and disclaims all liability for such use. Any person utilizing this instruction who is not a military or civilian employee of the United States of America should seek competent professional advice to verify and assume responsibility for the suitability of this information to their particular situation regardless of similarity to a corresponding Department of Defense or other government situation.